| Measurement Variable | Description | Timescale | Smallest Geographic Unit available | Measured or modeled? | Pros/Cons | Data Source Options |
| --- | --- | --- | --- | --- | --- | --- |
| Temperature | Degree or intensity of heat | Hourly, daily, weekly, monthly data. | Point data available from weather station locations – latitude and longitude. | Measured (min, max, average); but can be forecasted | Readily available, easy to understand and measure. Weather station data may not reflect the area of interest for surveillance or research | Local Climatological Data (noaa.gov); Online Data Request Submitted ¦ Climate Data Online (CDO) ¦ National Climatic Data Center (NCDC) (noaa.gov) |
| Heat Index | Combines relative humidity with actual air temperature | Daily | Local weather stations | Measured; but can be forecasted | Easy to understand | https://healthdata.gov/dataset/CDC-WONDER-Daily-Air-Temperatures-and-Heat-Index/etgt-szcw https://www.visualcrossing.com/weather/weather-data-services |
| Wet Bulb Globe Temperature (WBGT) | Combines temperature, humidity, wind, and solar radiation | Historical data; Forecasted available up to 26hrs | NWS designated weather zones | Measured | Challenging to measure, requires specialized equipment | National Weather Service (NWS) https://www.weather.gov/tsa/wbgt |