| Measurement Variable | Description | Pros/Cons of Measures | Timescale | Smallest Geographic Unit available | Measured or modeled? | Pros/Cons of Data Source | Data Source Options |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | Degree or intensity of heat | Pro: Easy to understand and measure Con: Temperature alone is not the whole story | Hourly, Daily, Weekly, Monthly | Measured at local weather stations, forecasted for any point | Measured (min, max, average); but can be forecasted | Con: Weather station data may not reflect the area of interest for surveillance or research | Local Climatological Data (noaa.gov); Online Data Request Submitted ¦ Climate Data Online (CDO) ¦ National Climatic Data Center (NCDC) (noaa.gov) |
| Heat Index | Combines relative humidity with actual air temperature | Pro: Easy to understand Con: Like temperature, measured in shade, so does not consider sunlight. | Hourly, Daily | Measured at local weather stations, forecasted for any point | Measured; but can be forecasted | Con: Weather station data may not reflect the area of interest for surveillance or research; may not be as easily available as temperature | https://ephtracking.cdc.gov/DataExplorer/ https://healthdata.gov/dataset/CDC-WONDER-Daily-Air-Temperatures-and-Heat-Index/etgt-szcw https://www.visualcrossing.com/weather/weather-data-services |
| Wet Bulb Globe Temperature (WBGT) | Combines temperature, humidity, wind, and solar radiation | Pro: Measure has multiple components to better reflect working conditions Con: Challenging to measure, requires specialized equipment, more challenging to interpret | Hourly, Daily | Measured at location; can be estimated elsewhere using satellite data | Measured, but can be forecasted | Con: Reliable or consistent data may not be readily available for all locations or time periods. | National Weather Service (NWS) https://www.weather.gov/tsa/wbgt |